# SENTINEL AI Disclosure Figures

Supplementary technical drawings for the Edge AI urban emergency detection prototype

The existing project circuit diagram is intentionally preserved in the original disclosure document. This appendix supplies the additional labeled and unlabeled figure sets requested for the complete specification.

# Figure 2. Labeled hardware wiring diagram

Figure 2: Edge-AI Multi-Sensor Urban Emergency Detection – Raspberry Pi 4 Hardware Wiring Diagram

# Figure 3. System architecture and response flow

Figure 3: Multimodal sensing, governed decision, operator workflow and audit record

# Connection table

| Component | Component pin | Physical pin | GPIO or signal | Purpose |
| --- | --- | --- | --- | --- |
| DHT22 | VCC / DATA / GND | 1 / 7 / 9 | 3.3V / GPIO4 / GND | Temperature and humidity |
| GY-87 / MPU6050 | 3.3V / GND / SDA / SCL | 1 / 6 / 3 / 5 | 3.3V / GND / GPIO2 / GPIO3 | Impact and motion over I2C |
| ADS1115 | VDD / GND / SDA / SCL / ADDR / A0 | 1 / 6 / 3 / 5 / GND / ADC | 3.3V / GND / GPIO2 / GPIO3 / 0x48 / A0 | ADC for MQ-2; use divider |
| MQ-2 | VCC / GND / AO / DO | 5V rail / GND / ADS1115 A0 / NC | 5V / GND / analog | Smoke and gas; never direct to GPIO |
| INMP441 | VDD / GND / SCK / WS / SD / L/R | 1 / GND / 12 / 35 / 38 / GND | 3.3V / GPIO18 / GPIO19 / GPIO20 | I2S audio |
| NEO-6M | VCC / GND / TX / RX | appropriate / GND / 10 / 8 | GPIO15 RX / GPIO14 TX | UART position |
| LEDs | R / Y / G anodes; cathodes | 29 / 31 / 33 / GND | GPIO5 / GPIO6 / GPIO13 | Each anode through 220–330Ω |
| Buzzer | VCC / GND / I/O | 3.3V / GND / 36 | GPIO16 PWM | Active-high or PWM module |
| SG90 | VCC / GND / signal | 5V / common GND / 32 | GPIO12 PWM0 | Separate suitable 5V supply recommended |
| USB webcam | USB | USB port | No GPIO | Video input |
| Reserved | GPIO26 | 37 | UNUSED | Must remain unconnected |

# Electrical notes

The Raspberry Pi GPIO header is 3.3 V logic. The MQ-2 heater is powered from 5 V and its analog output must be reduced with a suitable voltage divider before entering the 3.3 V ADS1115 input. The SG90 servo must use a suitable 5 V supply and share ground with the Raspberry Pi; its motor current should not be drawn through a GPIO pin. The drawing uses physical 40-pin header numbering and BCM GPIO names together.