SECTION A: TECHNICAL DETAIL REPORT

| 1. | Title of the Invention (should not exceed 15 words) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Edge-AI Multi-Sensor Urban Emergency Detection, Localization and Governed Response System |  |  |  |  |  |  |  |  |
| 2. | What is the Invention? (Should include Problem and Solution statement and general purpose in relation with your invention) |  |  |  |  |  |  |  |  |
|  | This invention is an edge-computing safety system for traffic intersections. Its purpose is to understand an emergency locally, using more than one type of evidence, instead of depending on a single camera, a single sensor, or a remote cloud service. In ordinary CCTV-based monitoring, an operator may have to notice the problem first, while cloud analytics can add communication delay and may stop working properly when the network is poor. A single detector can also be misleading; for example, exhaust smoke, dust, glare or a loud horn can look or sound like an emergency when considered on their own. The present prototype brings together three AI functions: an audio classifier for ambient sound, crash, horn and siren; a fire/smoke vision detector; and a vehicle detector for car, truck, bus and motorcycle. Their outputs are passed into the project sensor pipeline and deep-rule logic rather than being treated as final decisions by themselves. The project also supports separate model profiles, so experimental models can be tested without replacing the original legacy models. A Model Assurance layer has been added to show the active profile, live confidence, held-out F1 reliability, source of the training data, readiness level and classes that are not reliable enough for normal use. Because the current newly trained profile is based on synthetic/generated data, it is intentionally labelled RESEARCH_ONLY. A high confidence score therefore does not automatically mean that the model is ready for real-world traffic control. Human confirmation remains part of the operating concept. The wider invention is intended to evolve into a multimodal, explainable incident-management platform. Future embodiments can add temporal validation, dynamic risk scoring, unknown/out-of-distribution detection, confidence calibration, event correlation, device health, model drift, operator feedback, real-time camera and microphone streams, incident evidence packages, site maps and other traffic-intelligence functions. These extensions follow the same principle: the system should expose uncertainty, combine independent evidence and continue to operate locally even when external connectivity is unavailable. |  |  |  |  |  |  |  |  |
| 3. | Search Terms (Please provide key words and common industry phrases on the basis of which this invention could be connected with the prior art) |  |  |  |  |  |  |  |  |
|  | The invention is an offline-first edge-computing safety system for urban intersections. It addresses delayed and unreliable emergency detection caused by single-camera systems, isolated alarms, cloud dependence and unverified automatic actuation. A Raspberry Pi 4 combines acoustic, visual, thermal/humidity, gas/smoke, inertial and GPS evidence; produces a localized risk assessment; presents a governed response plan; and records an auditable incident package. Emergency-service contact remains operator-approved and the system never silently claims that a synthetic model is field validated. |  |  |  |  |  |  |  |  |
| 4. | Background of the Invention (Present state of Art) |  |  |  |  |  |  |  |  |
|  | a. What are the present technologies that exist in the field of your invention? The field already includes several mature technologies, but they are usually deployed as separate systems. These include CCTV and video analytics for traffic monitoring; inductive loops, radar and adaptive signal controllers for traffic flow; standalone fire, smoke, gas and environmental sensors; emergency-vehicle pre-emption systems using dedicated radio, infrared, GPS or transponder equipment; computer-vision models for vehicle, accident, fire and smoke detection; and acoustic classifiers for sirens or abnormal road sounds. There are also cloud-based command-centre platforms that collect information from different sites, and newer edge-AI products that run selected analytics near the camera. |  |  |  |  |  |  |  |  |
|  | b. What are the limitations of the same? The main limitation is that many existing systems solve only one part of the problem. A camera may detect smoke but may not know whether it is exhaust, dust or a genuine fire. An acoustic system may hear a loud sound but cannot always distinguish a horn, impact or siren reliably in a noisy junction. Cloud-dependent solutions introduce a communication path between the event and the decision, and may degrade during network loss. Dedicated ambulance-priority systems can require equipment to be fitted in the vehicle. In addition, a conventional AI dashboard often shows a confidence score without showing whether the model was trained on representative data, whether the input quality is poor, whether the model is drifting, or whether a second sensor agrees. These gaps can lead to false alarms, operator mistrust and poor accountability in safety-related use. |  |  |  |  |  |  |  |  |
| 5. | Prior Art (Please provide the details of the product or process by the help of which the problem solved by the invention was tackled before this invention. Please details of prior technologies, patents, or publications relevant to this invention) |  |  |  |  |  |  |  |  |
|  | Edge AI; Raspberry Pi 4; multimodal sensor fusion; urban emergency detection; crash detection; fire and smoke detection; acoustic event classification; INMP441; GY-87 MPU6050; MQ-2 gas sensor; ADS1115 ADC; DHT22; NEO-6M GPS; localization; traffic-corridor response; operator-in-the-loop; assurance; out-of-distribution detection; offline-first incident evidence; tamper-evident audit log; black-box recorder. |  |  |  |  |  |  |  |  |
| 6. | Has the work filled a major gap in the prior art? If yes, a brief description of this gap. |  |  |  |  |  |  |  |  |
|  | Yes. The project addresses the practical gap between isolated detectors and an integrated, explainable edge safety platform. Instead of treating one AI output as the truth, the architecture is designed to combine visual, acoustic and physical-sensor evidence, consider how reliable the active model is, and show the operator why an event has been raised. It can also continue local processing during loss of external connectivity. A second gap is AI deployment discipline. The project deliberately keeps the newly trained synthetic profile separate from the legacy profile and labels it RESEARCH_ONLY. That means the system can demonstrate a new model without silently treating synthetic-data performance as field performance. This separation of detection confidence, model reliability and deployment readiness is an important part of the invention concept. |  |  |  |  |  |  |  |  |
| 7. | What are the components of the Invention? (Explain the external and internal parts/blocks/systems-subsystems/ composition/steps of process of your invention). |  |  |  |  |  |  |  |  |
|  | Hardware / sensing embodiment: • Raspberry Pi 4 or another suitable edge computer. • Camera for road, vehicle, fire and smoke observation. • INMP441 I2S microphone or equivalent for acoustic events. • GY-87 IMU or equivalent inertial sensor for impact and motion information. • MQ-2 gas/smoke sensor through an ADS1115 ADC or equivalent environmental sensing. • DHT22 or equivalent temperature/humidity sensor. • GPS/time source such as Neo-6M where location/time tagging is required. • Prototype outputs can include signal lamps, a servo-operated barrier model, buzzer/siren and local display. Current software / AI components: • Audio emergency classifier: ambient, crash, horn and siren. • Fire/smoke vision detector. • Vehicle vision detector: car, truck, bus and motorcycle. • Selectable legacy and synthetic model profiles. • Sensor pipeline and deep-rule engine. • Raw image/audio prediction path. • Model Assurance module showing model profile, confidence, held-out F1, data source, readiness and unreliable classes. • Dashboard and APIs including /api/live and /api/assurance. • Dataset preparation, retraining and prediction utilities. • Local testing/simulation support and regression tests. Preferred advanced embodiment: The same architecture can include a multimodal fusion engine, temporal event state machine, 0-100 contextual risk score, explainable alert generator, unknown/OOD detection, confidence calibration, model registry, model rollback, model drift monitoring, device-health supervision, camera tamper and input-quality checks, incident database, evidence packages, operator feedback, active-learning queue, real-time camera/microphone streaming, vehicle tracking/counting, stopped-vehicle and wrong-way detection, zone-aware rules, notification adapters, digital site map, analytics, audit logging, offline store-and-forward synchronization, retention management and system self-test. These items are extensions of the disclosed architecture and should be described as future or preferred embodiments unless they are implemented and validated in the final frozen build. |  |  |  |  |  |  |  |  |
| 8. | How does the Invention work? Please describe the complete sequence of operation of the invention, including the inputs received, processing or operations performed, interaction between the relevant components, and the resulting output, action, or technical effect. (This portion of your invention disclosure deals with the working of your invention, the processes involved and the interactive actions taking place between the various components of your invention to give the desired output.) |  |  |  |  |  |  |  |  |
|  | 1. The sensors acquire synchronized observations at the edge. 2. Audio, camera and telemetry pipelines produce event candidates with timestamps and model identifiers. 3. A temporal fusion and deep-rule layer checks corroboration, signal quality, sensor availability and out-of-distribution conditions. 4. The governed decision layer assigns event type, zone, confidence and severity, and generates a response plan. 5. The operator reviews, acknowledges, confirms or rejects the plan; emergency-service notification is never automatic. 6. Approved traffic actions are sent to the actuator layer, including LEDs, buzzer, servo and configured corridor signals. 7. Incidents, predictions, evidence, operator actions, assurance state and outcomes are persisted locally through SQLite/WAL and exported as a hash-verified evidence package. 8. When connectivity returns, an outbox can synchronize records without blocking inference. |  |  |  |  |  |  |  |  |
| 9. | Novelty of the Invention/Inventive Step of the Invention (Please state the unique/ novel aspect of your invention and how it is different and/or better than the existing technologies): |  |  |  |  |  |  |  |  |
|  | The inventive contribution is the governed combination of: (i) heterogeneous low-cost sensors on a Raspberry Pi edge node; (ii) temporal, cross-modal corroboration with sensor-availability awareness; (iii) assurance states, model identity, usage restrictions and OOD reasons attached to every prediction; (iv) an operator-in-the-loop response contract that separates recommendation from authorization; (v) offline-first, tamper-evident incident evidence containing timeline, model versions, actions and file hashes; and (vi) safe actuator control with explicit power-domain and GPIO constraints. The novelty lies in this auditable arrangement and decision procedure rather than in claiming any individual sensor or neural network in isolation. |  |  |  |  |  |  |  |  |
| 10. | What are the advantages and improvements over the existing methods/devices? (Please list advantages and improvements against closest patent and non-patent documents/products in pipeline explaining technical advancement of present invention over the existing technologies) |  |  |  |  |  |  |  |  |
|  | Compared with a single-camera, single-sensor or cloud-only arrangement, the proposed system offers several technical improvements. Processing is performed close to the source, which reduces dependence on network round trips and allows continued local operation during connectivity loss. Multimodal confirmation gives the decision layer more context and can reduce false alarms that arise when one detector is fooled by exhaust, noise, glare or environmental conditions. Model Assurance makes the limitations of the active model visible instead of hiding them behind a confidence percentage. The legacy and experimental model sets can be kept separate, which makes evaluation safer. The architecture is also designed for traceability. Predictions, evidence, model version/profile and operator actions can be recorded as part of the incident history. In the advanced embodiment, the system adds temporal persistence, OOD handling, input-quality checks, model drift, device health, risk-based prioritization and duplicate suppression. These functions make the platform more suitable for engineering evaluation and controlled deployment than a detector that simply raises an alarm whenever one model crosses a threshold. |  |  |  |  |  |  |  |  |
| 11. | Do you have in your mind as a researcher/inventor some ideas as to what are the various other/alternative forms in which your Invention can be depicted or utilized, in addition to one basic embodiment that has been described above? (This will lay the foundation, for making your patent as broad as possible. These are called preferred embodiments or variants.) |  |  |  |  |  |  |  |  |
|  | Alternative embodiments include additional camera nodes, LoRa/Wi-Fi/Ethernet backhaul, other I2S microphones, calibrated gas sensors, thermal cameras, CAN/vehicle telemetry, a larger ADC, a cloud mirror, or a different edge computer. The safety contract, operator approval, evidence manifest and model-usage restriction remain applicable. The Raspberry Pi embodiment is preferred for the disclosed prototype. |  |  |  |  |  |  |  |  |
| 12. | Please provide final CAD images, line diagrams, and schematic images of the final prototype illustrating all the essential elements of the invention, and shortlist the major/primary drawings that need to be incorporated in the Complete Specification. (Please prepare images in two sets, one completely labeled to details and the second one without any description, legends/labeling or numbering. Include relevant Flow charts.) |  |  |  |  |  |  |  |  |
|  | The following supplementary drawings are provided in two sets as requested: a fully labeled set for technical specification and a clean set without descriptive labels for formal drawing use. The original project diagram remains preserved in the source disclosure. Fig. 2: Fully labeled Raspberry Pi 4 hardware wiring diagram Fig. 3: Clean wiring drawing without descriptive labels Fig. 4: System architecture and emergency response flow Key connection summary Component; Pin / signal; Pi physical pin; GPIO / purpose. DHT22; DATA; 7; GPIO4 / temperature-humidity. GY-87; SDA, SCL; 3, 5; GPIO2, GPIO3 / I2C. ADS1115; SDA, SCL, ADDR; 3, 5, GND; 0x48 / ADC. MQ-2; AO; ADS1115 A0; 5 V sensor; divider required. INMP441; SCK, WS, SD; 12, 35, 38; GPIO18, 19, 20 / I2S. GPS; TX, RX; 10, 8; GPIO15 RX, GPIO14 TX. LEDs; R, Y, G; 29, 31, 33; GPIO5, 6, 13. Buzzer / servo; I/O / signal; 36 / 32; GPIO16 PWM / GPIO12 PWM. Reserved; GPIO26; 37; UNUSED. |
| 13. | Brief description from the researcher/inventor in his own words explaining the images/ drawings. |  |  |  |  |  |  |  |  |
|  | I designed SENTINEL-AI so that an emergency is treated as a sequence of evidence and decisions rather than as one alarm. The device senses locally, checks whether different sensors agree, explains why it raised a risk, lets an authorized human approve the response, and preserves what happened for later review. This makes the prototype useful even when the network is unavailable and makes its limitations visible. |  |  |  |  |  |  |  |  |
| 14. | Please provide a complete description of the invention, including its working methodology and working examples illustrating the inventive aspects of the corresponding invention. |  |  |  |  |  |  |  |  |
|  | The invention is an edge-computing device and method for detecting, assessing and managing emergency-related conditions at a traffic junction. A Raspberry Pi 4 or equivalent edge computer receives camera, microphone and, where fitted, IMU, gas/smoke, temperature/humidity and GPS inputs. The present software contains three trained AI functions: an audio classifier for ambient, crash, horn and siren; a fire/smoke detector; and a vehicle detector for car, truck, bus and motorcycle. The outputs are passed through the project sensor pipeline and deep-rule logic together with available physical-sensor observations. Working example 1 - possible vehicle accident: if the audio model detects a crash while the camera confirms a vehicle and the physical sensors indicate abnormal impact or motion, the combined evidence is stronger than the audio result alone. The system can therefore raise an accident candidate with the supporting evidence. Working example 2 - possible fire: if smoke or fire is detected visually, the result can be checked against temperature, gas and other available context. If the evidence is weak or contradictory, the event can remain under review instead of immediately being treated as a confirmed fire. Working example 3 - model assurance: if the synthetic profile produces a high-confidence prediction, the dashboard still shows that the model is RESEARCH_ONLY because the training source is synthetic/generated. The operator can see the live confidence and the held-out reliability separately and is required to treat the result as decision support. Preferred advanced working example - correlated accident and fire: a vehicle is first detected, a crash sound follows, the vehicle then remains stationary and smoke appears several seconds later. A temporal/correlation engine can combine these observations into one evolving incident rather than four separate alerts. The risk score rises as independent evidence accumulates. If the input is unfamiliar, the OOD module can return UNKNOWN/REVIEW_REQUIRED rather than forcing the sample into a known class. The incident manager records the timeline, model versions, confidence, assurance state and operator actions in a local database, while optional synchronization can occur when connectivity is available. |  |  |  |  |  |  |  |  |
| 15. | Please provide details of experiments conducted and/or third-party validation data, if any, along with the mathematical equations, formulas, or calculations used in the invention, wherever the invention performs arithmetic calculations or mathematical operations. |  |  |  |  |  |  |  |  |
|  | Prototype validation includes unit and integration tests for the database migrations, incident IDs and merge windows, duplicate suppression, writer-queue resilience, evidence export and tamper detection, model checksum mismatch, role permissions and regression behavior. Simulation live-streams produce incident records and evidence manifests. Results are laboratory/prototype evidence; field performance, emergency dispatch integration and safety certification remain future validation steps. |  |  |  |  |  |  |  |  |
| 16. | Please provide a set of instructions or an exemplary set of code, including the code type, version, and any modifications or parameter tuning, where the invention primarily relies on software, code, or an algorithm. |  |  |  |  |  |  |  |  |
|  | Reference software setup and exemplary use: 1. Create a Python virtual environment and install the repository dependencies. 2. Place the trained weights in the configured model directory and select the required model profile. The synthetic profile must remain separate from the legacy model set. 3. Prepare datasets with src/training/prepare_new_dataset.py. 4. Train/retrain with src/training/train_new_dataset.py. 5. Run raw prediction using scripts/predict_new_dataset.py or the supported API path. 6. Start the dashboard/API using the project entry point and open http://127.0.0.1:8080. 7. GET /api/live returns live system/inference information and GET /api/assurance returns model-assurance information. 8. Image/audio inference outputs are forwarded into the sensor/deep-rule pipeline. 9. On Raspberry Pi or other edge hardware, connect the supported sensors/outputs using the project pin map and use the hardware drivers. Where physical hardware is unavailable during development, simulation must be clearly labelled. For a complete patent example, the final frozen repository should also list the Python version, package versions, model filenames/checksums, configured thresholds, hardware pin map, model profile configuration and exact command used to start the service. |  |  |  |  |  |  |  |  |
| 17. | Please provide details of the training data, including its source, type, and relevant characteristics, where the invention relates to a machine learning (ML) model. Also, specify the ML model used and explain how the ML model is trained, implemented, and utilized in the invention. |  |  |  |  |  |  |  |  |
|  | The project uses acoustic, visual and sensor telemetry data. Acoustic classes include ambient, crash, horn and siren; visual classes include fire, smoke and vehicle categories; telemetry includes temperature, humidity, inertial motion, gas level and GPS. Synthetic samples may be used for research augmentation, but the registry marks the profile RESEARCH_ONLY and stores unavailable evaluation metrics as UNVERIFIED. Every stored prediction carries model ID/version, assurance state and usage restriction. |  |  |  |  |  |  |  |  |
| 18. | Please provide details of any public disclosure done/ any publication made. |  |  |  |  |  |  |  |  |
|  | No public disclosure or publication is confirmed in the material provided for this form. Before submission, the applicant should check carefully whether the project or any inventive detail has appeared in a public GitHub repository, college demo, poster, hackathon, conference submission, paper, thesis repository, social-media post, public presentation or video. If any such disclosure occurred, the exact date, audience, URL/location and material disclosed should be recorded here because it may affect patent strategy. |  |  |  |  |  |  |  |  |
| 19. | What is the stage/level of development of the Invention? |  |  |  |  |  |  |  |  |
|  | a. At a basic conceptualization stage? No. The project is beyond the concept stage. The AI models, raw inference path, sensor/deep-rule integration, selectable model profiles, Model Assurance, dashboard APIs, training utilities and regression tests exist in a working software prototype. |  |  |  |  |  |  |  |  |
|  | b. Completed and results validated? The present integrated software prototype is complete enough for research and laboratory demonstration, but it should not yet be described as fully validated for live traffic operation. The dashboard/API is running and the main software paths have regression coverage. The new synthetic profile is deliberately RESEARCH_ONLY. Real-road/junction trials, environmental testing, calibration, field false-alarm measurement, fail-safe verification and independent/third-party evaluation are still required before operational deployment claims are made. |  |  |  |  |  |  |  |  |
| 20. | What are the aspects of your disclosure that you want to claim/monopolize? (Proposed Claims) |  |  |  |  |  |  |  |  |
|  | Advantages include low-latency local operation; resilience during network loss; multi-modal corroboration that can reduce single-sensor false alarms; explicit uncertainty and OOD handling; operator-controlled escalation; physical pin and power safety; synchronized incident timelines; hash-verified evidence; role-based permissions; and a hardware/software architecture that can be reproduced on low-cost edge hardware. |  |  |  |  |  |  |  |  |
| 21. | - Have you conducted novelty/inventiveness search for your invention? If yes, what are the databases /references used by you? What are the search results? |  |  |  |  |  |  |  |  |
|  | A preliminary, informal review has been made against broad categories such as emergency-vehicle priority systems, adaptive traffic signal control, cloud/edge video analytics, standalone acoustic detectors, YOLO-based fire/smoke and vehicle detection, and CNN-based sound classification. That review did not identify the same complete project-specific arrangement of multimodal edge inference, deep-rule/sensor fusion, model-assurance/readiness reporting, research-model separation, human confirmation, offline operation and evidence logging. No formal novelty or inventive-step opinion is available in the supplied material. Before filing, a structured search should be performed using Google Patents, WIPO PATENTSCOPE, Espacenet, the Indian Patent Advanced Search System and relevant IEEE/ACM/other technical publications, with the closest references documented in a claim-feature comparison table. |  |  |  |  |  |  |  |  |
| 22. | Do you feel that a person of “average” skill (not-extraordinary skill) in your area of technology would have arrived at your invention with existing knowledge in public domain? If no, what could be the reasons for the same? |  |  |  |  |  |  |  |  |
|  | The individual building blocks - object detection, audio classification, Raspberry Pi sensing, dashboards and rules - would be familiar to a skilled engineer. The inventive-step argument therefore should not rely on any one of those parts. It should rely on the particular interaction between them: multimodal confirmation, explicit uncertainty handling, model-readiness separation, human confirmation for research models, offline incident operation and an auditable chain of evidence. Whether that combination would have been obvious at the filing date is ultimately a legal and technical question that depends on the closest prior art. A final position should therefore be taken only after the formal patent search. |  |  |  |  |  |  |  |  |
| 23. | Kindly provide broad workable ranges for all parameters involved in the invention, including the working parameters and ranges applicable to each component utilized in the system. |  |  |  |  |  |  |  |  |
|  | Illustrative prototype ranges are: 3.3 V logic for Raspberry Pi GPIO/I2C/I2S; 5 V supply for MQ-2 heater and SG90 servo; DHT22 data on GPIO4; I2C at GPIO2/GPIO3; I2S on GPIO18/GPIO19/GPIO20; UART on GPIO14/GPIO15; LEDs on GPIO5/GPIO6/GPIO13 with 220–330 ohm resistors; buzzer PWM on GPIO16; servo PWM on GPIO12; ADS1115 address 0x48; MQ-2 AO through a divider into ADS1115 A0; GPIO26/physical pin 37 unused. Actual thresholds are configuration values validated during commissioning. |  |  |  |  |  |  |  |  |
| 24. | Commercialization Data (at least a minimum of 2 companies must be provided and preferably up to 5) |  |  |  |  |  |  |  |  |
|  | a. Kindly give the names and complete addresses of five different companies which might be interested in the commercial manufacture of this technology. Potential organizations for industry discussion include: 1. Kapsch TrafficCom - [COMPLETE CURRENT POSTAL ADDRESS TO BE VERIFIED]. 2. SWARCO - [COMPLETE CURRENT POSTAL ADDRESS TO BE VERIFIED]. 3. L&T Smart World / L&T Technology Services - [COMPLETE CURRENT POSTAL ADDRESS TO BE VERIFIED]. 4. Bosch Mobility - [COMPLETE CURRENT POSTAL ADDRESS TO BE VERIFIED]. 5. Global Traffic Technologies - [COMPLETE CURRENT POSTAL ADDRESS TO BE VERIFIED]. At least two complete addresses should be verified from official company sources immediately before submission rather than copied from an outdated source. |  |  |  |  |  |  |  |  |
|  | b. Kindly also enclose a short marketing profile of the technology highlighting the advantages vis-a-vis the prior-art technologies in this area. This technology is a modular Edge-AI safety platform for traffic intersections and similar monitored environments. Its main commercial value is that it does not depend on a single detector or continuous cloud processing. Visual, acoustic and sensor evidence can be combined locally, while Model Assurance tells the operator whether the active model is dependable enough to trust. That gives a system integrator a clearer path from research AI to controlled deployment. The architecture can run on compact edge hardware, reduce continuous transmission of raw media, continue core operation during connectivity loss, and retain incident evidence for review. It can also be expanded with risk scoring, temporal validation, device health, site maps, model drift and operator feedback without replacing the original model profile. |  |  |  |  |  |  |  |  |
| 25. | Provide available data or information on market potential/size, adoption trends, industry analysis, and projected growth. |  |  |  |  |  |  |  |  |
|  | The commercial opportunity sits across intelligent transportation systems, adaptive traffic management, smart-city safety, edge video analytics, emergency-vehicle priority and AI-assisted incident management. Industry adoption is moving toward local processing because of latency, bandwidth, resilience and privacy requirements, and toward explainable/assured AI because operators need to know when a model is uncertain. The project is aligned with those trends. Exact market size, CAGR and forecast values are not established by the project material supplied for this disclosure. Those figures should be added only from a current, citable market report at the time of submission, with the report title, publisher and year recorded. |  |  |  |  |  |  |  |  |
| 26. | Please provide the names of the inventors involved in the Invention |  |  |  |  |  |  |  |  |
|  | Inventor details: [TO BE COMPLETED BY APPLICANT]. For each inventor, provide the full legal name, registration/employee ID, department, specific inventive contribution and signature. Inventorship should be based on contribution to the inventive concepts being claimed; supervision, funding or routine coding alone does not automatically make a person an inventor. |  |  |  |  |  |  |  |  |
| 27. | User Information |  |  |  |  |  |  |  |  |
|  | Potential users are traffic control rooms, municipal emergency operations centres, campus/industrial safety teams, road operators, researchers and trained maintenance engineers. Direct operators are trained adults with role-specific permissions: Commander (full incident/system authority), Operator (traffic and acknowledgement), Engineer (sensors, calibration and models), and Viewer (read-only monitoring). Benefits include faster local assessment, explainable response, reduced dependence on cloud connectivity and recoverable evidence. |  |  |  |  |  |  |  |  |
|  | b. Age group of the user? The direct users are trained adult operators, engineers, supervisors and administrators. The road-safety benefit extends to road users of all ages. The system does not need to profile the age of people in the scene. |  |  |  |  |  |  |  |  |
|  | c. Expected benefits to the user? Faster local detection and assessment of emergencies; reduced dependence on cloud connectivity; fewer false alarms caused by a single sensor; clearer explanation of why an alert was raised; visibility of model reliability and readiness; continued local operation during network loss; structured incident evidence for investigation; and, after appropriate validation, a path toward safer emergency-response and traffic-priority functions. |  |  |  |  |  |  |  |  |
|  | d. Cost advantage when compared to available solutions The prototype can use commodity edge hardware and relatively low-cost sensors rather than requiring a large central server at every location. Local processing can also reduce continuous bandwidth and cloud-storage requirements. The modular architecture allows functions to be added by software and configuration. A firm lifecycle-cost saving should only be claimed after the final bill of materials, maintenance cost and commercial alternatives are benchmarked on the same basis. |  |  |  |  |  |  |  |  |
| 28. | What is Technology readiness level of your invention? (Tick the appropriate TRL) |  |  |  |  |  |  |  |  |
| Research |  |  |  | Development |  |  | Deployment |  |  |
| TRL 1 |  | TRL 2 | TRL 3 | TRL 4 | TRL 5 | TRL 6 | TRL 7 | TRL 8 | TRL9 |
| Basic Principles observed |  | Technology concept formulated | Experimental proof of concept | Technology validated in a lab | Technology validated in a relevant environment (Industrially relevant in case of key enabling technologies) | Technology demonstrated in a relevant environment (Industrially relevant in case of key enabling technologies) | System prototype demonstration in an operational environmen | System complete and qualified | Actual system proven in an operational environment (Competitive manufacturing in case of key enabling technologies, or in space) |
| Recommended present level: TRL 4-5, depending on the institution's interpretation. The integrated software prototype, trained models, APIs, Model Assurance and regression tests demonstrate laboratory validation, and the system is structured for Raspberry Pi/edge deployment. However, the new synthetic model profile remains RESEARCH_ONLY and a full real-junction trial has not yet been completed. Tick TRL 4 if the institution requires validation to remain strictly within a laboratory environment. Tick TRL 5 only if the assembled prototype has been demonstrated in a representative traffic-junction model or comparable relevant environment. TRL 6 should be used only after the integrated prototype is demonstrated in a relevant physical environment, and TRL 7 only after an operational-environment prototype demonstration. |  |  |  |  |  |  |  |  |  |
| 29. | Any additional notes or remarks. |  |  |  |  |  |  |  |  |
|  | Additional notes: the supplied first/project circuit diagram is intentionally preserved. This completed disclosure adds the faculty-style supplementary labeled schematic, clean drawing set, system architecture flow, connection table and technical notes. The MQ-2 is not connected directly to a Raspberry Pi GPIO; the SG90 has a suitable 5 V source and common ground; USB camera uses a USB port; and GY-87 auxiliary pins VCC_IN, FSYNC, INTA and DRDY remain unconnected unless the selected module requires them. Patentability, inventorship and legal claim scope should be reviewed by the institution's IP office. |  |  |  |  |  |  |  |  |